- DFT:

Without using built in fft functions

clc; clear;

x = input('Enter x[n]: ');

x = x(:).';                % ensure row vector

N = length(x);

n = 0:N-1; k = 0:N-1;

X = x * exp(-1j*2*pi/N).^(n'*k);   % DFT computation

disp('X[k] = ');

disp(X);

subplot(3,1,1); stem(n,x);

subplot(3,1,2); stem(k,abs(X));

subplot(3,1,3); stem(k,angle(X));

Using fft built in functions:

clc; clear;

x = input('x[n]= ');

X = fft(x);

disp(X);

n = 0:length(x)-1;

subplot(3,1,1); stem(n,x);

subplot(3,1,2); stem(n,abs(X));

subplot(3,1,3); stem(n,angle(X));

DFT USING MATRIX METHOD:

clc; clear;

x = input('x[n]= ');

N = length(x);

n = 0:N-1; k = n';

X = exp(-1j*2*pi/N).^(k*n) * x(:);

disp(X.');

subplot(3,1,1); stem(n,x);

subplot(3,1,2); stem(n,abs(X));

subplot(3,1,3); stem(n,angle(X));

- LINEAR AND CIRCULAR CONVOLUTION USING DFT:

LINEAR:

clc; clear;

x=input('x[n]= '); lx=input('lx= ');

h=input('h[n]= '); lh=input('lh= ');

y=conv(x,h); disp(y);

n1=lx:lx+length(x)-1;

n2=lh:lh+length(h)-1;

n3=lx+lh:lx+lh+length(y)-1;

subplot(3,1,1); stem(n1,x);

subplot(3,1,2); stem(n2,h);

subplot(3,1,3); stem(n3,y);

CIRCULAR:

x1=input('x1=[ ]: '); x2=input('x2=[ ]: ');

N=max(length(x1),length(x2));

y=real(ifft(fft(x1,N).*fft(x2,N))); disp(y);

n=0:N-1;

subplot(3,1,1); stem(n,[x1 zeros(1,N-length(x1))]);

subplot(3,1,2); stem(n,[x2 zeros(1,N-length(x2))]);

subplot(3,1,3); stem(n,y);

- DFT USING DIT

clc; clear;

x=input('x= '); N=input('N= ');

x=[x zeros(1,N-length(x))]; x=bitrevorder(x);

for s=1:log2(N)

m=2^s; W=exp(-1j*2*pi/m);

for k=1:m:N

for j=0:m/2-1

t=W^j*x(k+j+m/2);

x([k+j k+j+m/2])=[x(k+j)+t x(k+j)-t];

end

end

end

disp(x); disp(fft(x,N));

DFT USING DIF:

Above code

- FIR FLITER

RECTANGULAR WINDOW:

clc; clear;

N=input('Enter filter length N: ');

fc=input('Enter cutoff frequency (0–0.5): ');

n=0:N-1; wc=2*pi*fc;

hd=sin(wc*(n-(N-1)/2))./(pi*(n-(N-1)/2)); hd((N+1)/2)=2*fc;

h=hd.*rectwin(N)'; freqz(h,1);

title('FIR Filter using Rectangular Window');

HAMMING WINDOW:

clc; clear;

N=input('Enter filter length N: ');

fc=input('Enter cutoff frequency (0–0.5): ');

n=0:N-1; wc=2*pi*fc;

hd=sin(wc*(n-(N-1)/2))./(pi*(n-(N-1)/2)); hd((N+1)/2)=2*fc;

h=hd.*hamming(N)'; freqz(h,1);

title('FIR Filter using Hamming Window');

HANNING WINDOW:

clc; clear;

N=input('Enter filter length N: ');

fc=input('Enter cutoff frequency (0–0.5): ');

n=0:N-1; wc=2*pi*fc;

hd=sin(wc*(n-(N-1)/2))./(pi*(n-(N-1)/2)); hd((N+1)/2)=2*fc;

h=hd.*hanning(N)'; freqz(h,1);

title('FIR Filter using Hanning Window');

- IIR FILTER USING IIT AND BT

BUTTERWORTH LOWPASS FILTER

clc; clear; close all;

wp=input('wp='); ws=input('ws='); Rp=input('Rp='); As=input('As=');

[N,wn]=buttord(wp/pi,ws/pi,Rp,As); [b,a]=butter(N,wn);

[H,w]=freqz(b,a,512);

subplot(2,2,1),plot(w/pi,abs(H)),title('Mag');

subplot(2,2,2),plot(w/pi,20*log10(abs(H))),title('dB');

subplot(2,2,3),plot(w/pi,angle(H)),title('Phase');

subplot(2,2,4),grpdelay(b,a,512),title('Grp Delay');

CHEBYSHEV FILTER:

clc; clear; close all;

wp=input('wp='); ws=input('ws='); Rp=input('Rp='); As=input('As=');

if(wp>pi||ws>pi), error('wp, ws < pi'); end

[N,wn]=cheb1ord(wp/pi,ws/pi,Rp,As);

[b,a]=cheby1(N,Rp,wn,'low');

[H,w]=freqz(b,a,512);

subplot(2,2,1),plot(w/pi,abs(H)),title('Mag');

subplot(2,2,2),plot(w/pi,20*log10(abs(H))),title('dB');

subplot(2,2,3),plot(w/pi,angle(H)),title('Phase');

subplot(2,2,4),grpdelay(b,a,512),title('Grp Delay');

- QUANTIZATION EFFECTS IN DIGITAL FILTER DESIGN

CO-EFFICIENT QUANTIZATION EFFECTS ON THE

FREQUENCY REPSONSE OF A DIRECT FORM IIR FILTER

clc; clear; close all;

N=input('N= '); wc=input('wc= '); Ap=input('Ap= '); As=input('As= ');

[b,a]=ellip(N,Ap,As,wc); [H,w]=freqz(b,a,512);

bq=round(b*32)/32; aq=round(a*32)/32;

[Hq,~]=freqz(bq,aq,512);

plot(w/pi,20*log10(abs(H)),'b',w/pi,20*log10(abs(Hq)),'r--'); grid on;

xlabel('\omega/\pi'); ylabel('Gain(dB)');

title('Blue: Original | Red: Quantized');

figure; zplane(b,a);

CO-EFFICIENT QUANTIZATION EFFECTS ON THE

FREQUENCY REPSONSE OF A CASCADE IIR FILTER

clc; clear; close all;

N=input('N= '); wc=input('wc= '); Ap=input('Ap= '); As=input('As= ');

[b,a]=ellip(N,Ap,As,wc);

[H,w]=freqz(b,a,512);

bq=round(b*32)/32; aq=round(a*32)/32;

[Hq,~]=freqz(bq,aq,512);

plot(w/pi,20*log10(abs(H)),'b',w/pi,20*log10(abs(Hq)),'r--'); grid on;

xlabel('\omega/\pi'); ylabel('Gain(dB)');

title('Blue: Original | Red: Quantized');

figure; zplane(b,a);

- DECIMATION:

clc; clear; close all;

x=input('x= '); ni=input('Start index= '); M=input('Downsample factor M= ');

n=ni:ni+length(x)-1;

m=ni:M:n(end);

y=x(1:M:end);

subplot(2,1,1); stem(n,x); title('x[n]'); xlabel('n');

subplot(2,1,2); stem(m,y); title('y[m]=x[Mn]'); xlabel('m');

INTERPOLATION:

clc; clear; close all;

x=input('x= '); ni=input('Start index= '); L=input('Upsample factor L= ');

n=ni:ni+length(x)-1;

y=zeros(1,length(x)*L); y(1:L:end)=x;

m=ni*L:ni*L+length(y)-1;

subplot(2,1,1); stem(n,x); title('x[n]'); xlabel('n');

subplot(2,1,2); stem(m,y); title('y[m]=x[n/L]'); xlabel('m');

- ECG SIGNAL:

clc; clear; close all;

fs = input('Enter fs: ');

x = input('Enter ECG samples [ ]: ');

t = (0:length(x)-1)/fs;

[p,l] = findpeaks(x,'MinPeakHeight',0.5);

plot(t,x,'b',t(l),p,'ro'); grid on;

xlabel('Time(s)'); ylabel('Amplitude'); title('ECG Signal');

if length(l)>1

disp(['Heart Rate = ', num2str(60/mean(diff(t(l)))), ' bpm']);

else

disp('Not enough peaks');

end

- EEG SIGNAL:

clc; clear; close all;

fs = input('Enter sampling frequency (Hz): ');

eeg = input('Enter EEG signal samples [ ]: ');

t = (0:length(eeg)-1)/fs;

delta = bandpass(eeg,[0.5 4],fs);

theta = bandpass(eeg,[4 8],fs);

alpha = bandpass(eeg,[8 13],fs);

beta  = bandpass(eeg,[13 30],fs);

subplot(5,1,1); plot(t,eeg); title('Original EEG Signal');

subplot(5,1,2); plot(t,delta); title('Delta (0.5–4 Hz)');

subplot(5,1,3); plot(t,theta); title('Theta (4–8 Hz)');

subplot(5,1,4); plot(t,alpha); title('Alpha (8–13 Hz)');

subplot(5,1,5); plot(t,beta);  title('Beta (13–30 Hz)');

xlabel('Time (s)');

10.Arduino FFT

#include <arduinoFFT.h>

arduinoFFT FFT;

const uint16_t N = 128;

const double Fs = 1000, F = 100;

double R[N], I[N];

void setup() { Serial.begin(115200); }

void loop() {

for (int i = 0; i < N; i++) {

R[i] = 100 * sin(2 * PI * F * i / Fs);

I[i] = 0;

}

FFT.Windowing(R, N, FFT_WIN_TYP_HAMMING, FFT_FORWARD);

FFT.Compute(R, I, N, FFT_FORWARD);

FFT.ComplexToMagnitude(R, I, N);

for (int i = 0; i < N / 2; i++) {

Serial.print((i * Fs) / N);

Serial.print(" ");

Serial.println(R[i]);

}

Serial.println();

delay(1000);

}

Linear Arduino

int x[] = {1,2,3,4}, h[] = {1,1,1}, y[20];

int L = sizeof(x)/sizeof(x[0]), M = sizeof(h)/sizeof(h[0]), N;

void setup() {

Serial.begin(115200);

N = L + M - 1;

for (int n = 0; n < N; n++) {

y[n] = 0;

for (int k = 0; k < L; k++)

if (n - k >= 0 && n - k < M) y[n] += x[k] * h[n - k];

Serial.print(n); Serial.print(" "); Serial.println(y[n]);

}

}

void loop() {

for (int i = 0; i < N; i++) Serial.println(y[i]);

Serial.println();

delay(1000);

}

FIR Arduino

const int N=128,M=5;

float x[N],y[N],h[M]={0.2,0.2,0.2,0.2,0.2};

void setup(){

Serial.begin(115200);

for(int n=0;n<N;n++){

x[n]=50*sin(2*PI*5*n*0.01)+random(-10,10);

y[n]=0;

for(int k=0;k<M;k++) if(n-k>=0) y[n]+=h[k]*x[n-k];

}

for(int i=0;i<N;i++){ Serial.print(x[i]); Serial.print(" "); Serial.println(y[i]); }

}

void loop(){

for(int i=0;i<N;i++) Serial.println(y[i]);

Serial.println();

delay(1000);

}